[image: image1.png]Product Specifications

FFFFF




Bolt
washer
lock washer
Overall length in mm
Shoulder length in

mm
Thread length in

mm
number of bolts for part
Where it goes

Fan Mount bolt
8 cap
yes
6x56
6x39
6x17
4
to the four rubber cooling fan mountings 

Eng. Shroud Right
2

6x22
6x5
6x17
2
to right side engine cylinder shroud

Eng. Shroud Right & Left
2

10x22
0
10x22
2
to right and left sides engine cylinder upper rear shroud

Eng. Shroud 

Left
1

8x22
8x5
8x17
1
to left side front upper shroud

Cam shaft housing
plastic

bushing

6x29
9x14
6x15
4
Coupe Valve cover, special bolt

dia. 9mm shoulder x 6mm thread

"
"

6x35
9x20
6x15
4
Sedan Valve cover, special bolt 

dia. 9mm shoulder x 6mm thread

"


6x36
6x19
6x17
1
Coupe left cam housing cover 

"


6x25
6x8
6x17
3
Coupe left cam housing cover 

"


6x25
6x8
6x17
4
Sedan left cam housing cover 

"


6x32
6x15
6x17
1
Both right cam housing cover

"


6x45
6x28
6x17
3
Both right cam housing cover

"
1

6x130
6x112
6x18
1
Center front of housing

Cylinder Head
8-

10id 




8
6 nuts to head studs

2 cap nuts to two inside rear studs

"


6x25
6x8
6x17
1
Rear of head to Cam housing

Cylinder


6x28
6x11
6x17
1
Rear of head to Cylinders

"


6x32
6x15
6x17
3
1 Front, 2 Rear of Cylinders

Bolt
washer
lock washer
Overall length in mm
Shoulder length in

mm
Thread length in

mm
number of bolts for part
Where it goes

Clutch Cover


6x45
6x28
6x17
2
Clutch cover (short bolts)

"


6x50
6x33
6x17
2
Clutch cover (long bolts)

Drive Chain Cover


6x45
6x15
6x17
8
Sides and top of cover

Drive Chain Cover


6x36
6x19
6x17
4
lower 4 bolts

Bottom Case to upper case bolts 
4

8x125
8x105
8x20
4
Bottom front 4 bolts (with seal washer)

"


8x68
8x38
8x30
4
Bottom Rear 4 bolts 

"
3

6x90
6x73
6x17
3
Bottom Center 3 bolts (with seal washer)

"
1

6x50
6x33
6x17
1
Bottom Center 1 bolt (with seal washer) 

"


6x50
6x33
6x17
2
Bottom Center (outside forward)

"


6x55
6x38
6x17
2
Bottom Center (outside rear)

"


6x50
6x33
6x17
2
Bottom front inner to center line

"


6x60
6x43
6x17
2
Bottom front outer to center line

Top case center to bottom case
1

6x95
6x78
6x17
1
Center of top case (behind Hydraulic Tensioner).

Speedo Drive gear housing
1 seal

washer

6x12
1
6x6
1

special
Pin bolt, special bolt with pinning end to hold housing in place

Reverse Spring Cover


6x14
0
6x14
2
Reverse lever tension spring cover

Shift shaft ball spring retaining bolt
1

14x15
0
14x15
1
Shift shaft check ball and spring retaining bolt, at side of shift shaft housing. 

Reverse lever ball spring retaining bolt
1

14x17
0
14x17
1
To the side of the lever Tension spring cover. 

Points breaker plate to Dist. 
2, 10 id

15 od 

6x12
0
6x12
2
Use bolts to replace the phillips head screws holding the points breaker plate in place. 

Bolt
washer
lock washer
Overall length in mm
Shoulder length in

mm
Thread length in

mm
number of bolts for part
Where it goes

Center bolt to advancer assy
 6.5 id

19 od

6x14
0
6x14
1
Case Harden bolt to hold advancer assy to cam. 

Tach Cable retaining bolt
1

5x14
0
5x14
1
On Left cam side cover, retaining bolt for Tach. cable. 

Oil Drain Plug
1

20x14
0
20x14
1
To oil drain outlet. 

Total bolts for Engine Cases, Fan, Shrouds, Head nuts and Misc. washers =128 items.

